Material and Method

Chapter three: Material and method

3.1 Material:

3.1.1 Patients:

A total of 8 patients ( male , female ) with average age 42 , four patients  for  brain examination and four to abdomen examination ,under CT scan with 64 detectors and 8 detectors .

3.1.2 Machine:

| Slice NO. | Manufacture | Model | Year installation | Detector type |
| --- | --- | --- | --- | --- |
| 8 slice | General electric | Aquilion | 2010 | 8 rows |
| 64 slice | Toshiba | Aquilion | 2010 | 64 rows |

3.2 Methods:

3.2.1 Technique:

For brain scan by “8 slice” 8_10 mm  slice thickness “64 slice “ 3_5  mm slice thickness for axial mode , for spiral mode multi detector 0.5mm slice thickness .Axial cuts from the base of the skul to vertex parallel to the radiographic base line .

For abdomen scan by “8 and 64 slice” 5mm slice thickness .Axial cuts all the abdomen .

All multi detector CT scanner scan with 0.5mm then reconstruct the images according to the selected protocol “ 2mm ,3mm ,5mm .etc “

When the pation lies in correct position we use spiral technique , the advantage of spiral technique was , short scan time and low dose to the patient .the low dose in spiral technique depend on some factors ( mAs ,Kv ,pitch slice thickness) .

3.3 Data Collection :

Date were collected using a sheet for all patients in order to maintain consistency of the information form display  . A data collection sheet was designed to evaluate the patient dose and the radiation related factor .the collected data include tube voltage , tube current  , CT dose index (CTDI) ,and  dose length product (DLP).

3.4 Image Evaluation:

All images for abdomen and brain are a visually evaluation the quality, noise, contrast and artifacts by 30 ridiolgest.